Rescue Line Robot

Software Documentation

Melbourne High School

RoboCup Junior Rescue Line, Open

September 2026

# Overview

This document explains the software running on the rescue line robot. The robot is built around a Raspberry Pi 5, two cameras and four brushless motors. One camera faces down and follows the black line on the course. The other camera faces forward and looks for obstacles. Green markers next to the line tell the robot when to turn, and two markers together, one on each side, tell it to make a U-turn.

The code is organised into two folders. The vision folder handles everything the cameras see: finding the line, finding markers, finding obstacles. The control folder handles motor commands and the decision making that ties the vision results into actual driving. main.py is the entry point that starts everything up in the right order and shuts it down cleanly when a run ends.

The six files below are the ones that run during an actual competition attempt. Calibration tools, one-off test scripts and the physical start switch are separate utilities used during setup and are not included here, since they are not part of the logic that runs while the robot is on the course.

Six states control the robot at any given moment: following the line normally, recovering when only part of the line is visible, searching when the line is lost completely, checking a green marker at an intersection, carrying out a turn at that marker, and arcing around an obstacle. Every state eventually returns to line following once its job is done.

## main.py

Entry point

main.py is what actually gets run to start the robot, with the command python3 main.py. It reads a small set of command line flags so the code could be tested without motors or without cameras while the rest of the robot was still being assembled, for example python3 main.py --stub --no-cam runs the state machine on its own with no hardware attached at all.

On a normal run it loads any saved camera calibration from calibration.json, creates the motor driver, starts the two camera threads, waits for them to settle, then hands control over to the state machine. When the run ends, whether from Ctrl-C or the state machine finishing on its own, the finally block stops the motors and shuts the cameras down so nothing is left running.

1  """

2  main.py

3  =======

4  Entry point for the Rescue Line robot.

5

6  Hardware:

7      - Raspberry Pi 5

8      - 2x Pi Camera Module V2 (CSI - CAM1=line/downward, CAM0=forward)

9        (matches LINE_CAM=1, FWD_CAM=0 in vision/vision_thread.py)

10      - 4x RoboMaster M2006 P36 via Powerful BLDC Driver boards (I2C)

11      - I2C1 on Pi GPIO pins 3 (SDA) and 5 (SCL) at 1MHz

12

13  Run modes:

14      python3 main.py              # real hardware

15      python3 main.py --stub       # stub motors, real cameras (safe test)

16      python3 main.py --stub --no-cam  # no hardware at all (pure logic test)

17

18  Ctrl-C stops everything cleanly.

19

20  Before first run:

21      1. Add to /boot/firmware/config.txt:

22             dtoverlay=i2c1,pins_2_3,baudrate=1000000

23      2. Install motor library:

24             pip install git+https://github.com/Aw3someAndrew/SteelBar_CircuitPython_powerful_bldc_driver.git

25      3. Install camera library:

26             sudo apt install python3-picamera2 -y

27      4. Set I2C addresses in control/motors.py to match your boards

28      5. Run calibrate.py to tune vision thresholds for competition venue

29      6. Run calibrate_once.py to (re)generate calibration_offsets.json -

30         motors.py ONLY reads this file, it never calibrates live. Rerun

31         calibrate_once.py after any power loss to the driver boards, or

32         after swapping/rewiring a motor.

33  """

34

35  import argparse

36  import logging

37  import os

38  import sys

39  import time

40

41  # calibration.json lives next to this script, NOT in whatever CWD the process

42  # was launched from (systemd / SSH from another dir would silently miss it).

43  CALIBRATION_PATH = os.path.join(os.path.dirname(os.path.abspath(__file__)),

44                                  "calibration.json")

45

46  # When started with no interactive terminal (e.g. by switch_watcher.py at

47  # boot), we can't block on input() -- it would hang forever (or raise

48  # EOFError) with stdin redirected from /dev/null under systemd. Give this

49  # long a window to place the robot on the tile instead, then continue.

50  NO_TTY_CALIBRATION_DELAY_S = 5

51

52  logging.basicConfig(

53      level=logging.INFO,

54      format="%(asctime)s  %(levelname)-7s  %(message)s",

55      datefmt="%H:%M:%S",

56  )

57  log = logging.getLogger(__name__)

58

59

60  def main():

61      parser = argparse.ArgumentParser(description="Rescue Line robot")

62      parser.add_argument("--stub",    action="store_true",

63                          help="Use stub motors instead of real BLDC drivers")

64      parser.add_argument("--no-cam", action="store_true",

65                          help="Skip cameras entirely (stub vision)")

66      parser.add_argument("--debug",  action="store_true",

67                          help="Enable verbose vision debug output")

68      args = parser.parse_args()

69

70      from control.shared_vision import SharedVision

71      from vision.line_detector import load_calibration

72

73      # Apply any on-site calibration saved by calibrate.py, before any

74      # LineDetector is constructed. Without this, pressing "s" in calibrate.py

75      # tunes nothing that the actual robot run ever sees.

76      load_calibration(CALIBRATION_PATH)

77      from control.state_machine  import StateMachine

78

79      shared = SharedVision()

80

81      # --- Motor driver ---

82      if args.stub:

83          from control.motors import StubMotorDriver

84          motors = StubMotorDriver(verbose=args.debug)

85          log.info("Using StubMotorDriver")

86      else:

87          try:

88              from control.motors import BLDCMotorDriver

89              motors = BLDCMotorDriver()

90              log.info("BLDCMotorDriver initialised")

91          except Exception as e:

92              log.error("BLDC init failed: %s", e)

93              log.error("If this is a missing/incomplete calibration_offsets.json, "

94                         "run calibrate_once.py first. Run with --stub to test "

95                         "without motors.")

96              sys.exit(1)

97

98      # --- Vision threads ---

99      vision = None

100      if not args.no_cam:

101          from vision.vision_thread import VisionThread

102          vision = VisionThread(shared, debug=args.debug)

103

104          # Calibration window - robot should be placed on tile before calibrating.

105          # This ensures the first calibration frame is a real tile, not a hand or desk.

106          print("\n" + "="*50)

107          if sys.stdin.isatty():

108              print("Place robot on the WHITE TILE, then press ENTER to calibrate cameras.")

109              print("="*50)

110              input()

111          else:

112              # No interactive terminal -- e.g. started by switch_watcher.py at

113              # boot. Assume the Robot Handler already placed the robot on the

114              # tile per competition setup, give a short window to finish that,

115              # then continue rather than blocking forever on input().

116              print(f"No terminal attached - place robot on the WHITE TILE now, "

117                    f"calibrating cameras in {NO_TTY_CALIBRATION_DELAY_S}s...")

118              print("="*50)

119              log.warning("No interactive terminal - skipped ENTER prompt, "

120                          "waited %ds instead", NO_TTY_CALIBRATION_DELAY_S)

121              time.sleep(NO_TTY_CALIBRATION_DELAY_S)

122

123          vision.start()

124          log.info("Waiting 1s for cameras to settle...")

125          time.sleep(1.0)

126      else:

127          log.info("--no-cam: running without cameras")

128

129      # --- State machine ---

130      sm = StateMachine(shared, motors)

131

132      # --- Run ---

133      log.info("Starting control loop - Ctrl-C to stop")

134      try:

135          sm.run()

136      finally:

137          if vision:

138              vision.stop()

139          motors.stop()

140          if hasattr(motors, "cleanup"):

141              motors.cleanup()

142          log.info("Shutdown complete")

143

144

145  if __name__ == "__main__":

146      main()

147

## control/motors.py

Motor driver

motors.py talks to the four RoboMaster M2006 motors through their Powerful BLDC Driver boards over I2C. The PID constants, current limit and calibration values in this file were found through direct testing on the robot rather than taken from the manufacturer's example code, since the example values did not turn all four motors reliably.

The class exposes simple methods such as drive, rotate_left, rotate_right, drive_straight and stop, so the rest of the code never has to deal with I2C registers directly. A StubMotorDriver class with the same methods is also included, so the control loop can be run and tested on a laptop with no motors connected, by printing the commands instead of sending them.

1  """

2  motors.py

3  =========

4  Motor driver for 4x RoboMaster M2006 P36 brushless motors via I2C, using

5  the Powerful BLDC Driver boards.

6

7  Kept deliberately simple: BLDCMotorDriver below is a straight port of

8  test_forwardv2.py's setup_motor() - the only script confirmed to actually

9  drive the robot correctly - wrapped so state_machine.py can use it through

10  the same MotorDriver interface as StubMotorDriver. No calibration file,

11  no firmware-version check, no position PID, no double-apply of constants.

12

13  Sign convention (proven by test_forwardv2.py):

14      Forward = right side gets +speed, left side gets -speed.

15

16  IMPORTANT - calibration values are volatile:

17      elec_angle_offset / sincos_centre in MOTOR_CONFIG below reset if the

18      driver boards lose power. If the boards get power-cycled, re-run

19      calibrate_once.py (or re-verify test_forwardv2.py still drives

20      straight) and paste the new values into MOTOR_CONFIG here.

21

22  I2C1 on Pi GPIO pins 3 (SDA) / 5 (SCL), 1MHz - set in

23  /boot/firmware/config.txt:

24      dtoverlay=i2c1,pins_2_3,baudrate=1000000

25  """

26

27  import time

28  import logging

29  from abc import ABC, abstractmethod

30

31  log = logging.getLogger(__name__)

32

33  # Same values as test_forwardv2.py's MOTOR_CONFIG - proven to work.

34  MOTOR_CONFIG = {

35      "LEFT_FRONT":  {"address": 0x20, "elec_angle_offset": 1166513920, "sincos_centre": 1236},

36      "LEFT_REAR":   {"address": 0x1B, "elec_angle_offset": 1185746176, "sincos_centre": 1237},

37      "RIGHT_FRONT": {"address": 0x19, "elec_angle_offset": 1540904960, "sincos_centre": 1250},

38      "RIGHT_REAR":  {"address": 0x1A, "elec_angle_offset": 2142859264, "sincos_centre": 980},

39  }

40

41  SPEED_LIMIT     = 546133333   # driver-side hard cap (test_forwardv2.py proven)

42  MAX_SPEED_UNITS = 25000000   # what 100% maps to == test_forwardv2.py DRIVE_SPEED

43

44

45  # ---------------------------------------------------------------------------

46  # Abstract base

47  # ---------------------------------------------------------------------------

48

49  class MotorDriver(ABC):

50

51      @abstractmethod

52      def set_speeds(self, left: float, right: float) -> None:

53          """left, right: -100.0 (full reverse) .. 0 .. +100.0 (full forward)."""

54

55      def drive(self, base_speed: float, turn: float) -> None:

56          """

57          Differential drive. turn > 0 → turn right, turn < 0 → turn left.

58          Values in -100..100 percentage range.

59          """

60          left  = max(-100.0, min(100.0, base_speed + turn))

61          right = max(-100.0, min(100.0, base_speed - turn))

62          self.set_speeds(left, right)

63

64      def stop(self) -> None:

65          self.set_speeds(0.0, 0.0)

66

67      def rotate_left(self, speed: float = 30.0) -> None:

68          self.set_speeds(-speed, speed)

69

70      def rotate_right(self, speed: float = 30.0) -> None:

71          self.set_speeds(speed, -speed)

72

73      def drive_straight(self, speed: float = 50.0) -> None:

74          self.set_speeds(speed, speed)

75

76

77  # ---------------------------------------------------------------------------

78  # Real I2C driver - direct port of test_forwardv2.py

79  # ---------------------------------------------------------------------------

80

81  class BLDCMotorDriver(MotorDriver):

82      """

83      Controls 4x M2006 P36 motors via the Powerful BLDC Driver boards over

84      I2C. Same init sequence, PID constants, and calibration values as

85      test_forwardv2.py.

86

87      Install:

88          pip install git+https://github.com/Aw3someAndrew/SteelBar_CircuitPython_powerful_bldc_driver.git

89      """

90

91      def __init__(self):

92          import board

93          import busio

94          from steelbar_powerful_bldc_driver import PowerfulBLDCDriver

95

96          self._i2c = busio.I2C(board.SCL, board.SDA)

97

98          log.info("[BLDC] Initialising motors on I2C1...")

99          self._lf = self._setup_motor(PowerfulBLDCDriver, MOTOR_CONFIG["LEFT_FRONT"])

100          self._lr = self._setup_motor(PowerfulBLDCDriver, MOTOR_CONFIG["LEFT_REAR"])

101          self._rf = self._setup_motor(PowerfulBLDCDriver, MOTOR_CONFIG["RIGHT_FRONT"])

102          self._rr = self._setup_motor(PowerfulBLDCDriver, MOTOR_CONFIG["RIGHT_REAR"])

103          log.info("[BLDC] All motors initialised")

104

105      def _setup_motor(self, driver_cls, cfg):

106          m = driver_cls(self._i2c, cfg["address"])

107          m.set_current_limit_foc(65536)              # 1 amp

108          m.set_id_pid_constants(1100, 150)

109          m.set_iq_pid_constants(1100, 150)

110          m.set_speed_pid_constants(2.5e-2, 2.5e-4, 2e-2)

111          m.set_position_region_boundary(250000)

112          m.set_ELECANGLEOFFSET(cfg["elec_angle_offset"])

113          m.set_SINCOSCENTRE(cfg["sincos_centre"])

114          m.set_speed_limit(SPEED_LIMIT)

115          m.configure_operating_mode_and_sensor(3, 1)  # FOC, sin/cos encoder

116          m.configure_command_mode(12)                  # speed command mode

117          m.clear_faults()

118          return m

119

120      def _pct_to_units(self, pct: float) -> int:

121          units = int((pct / 100.0) * MAX_SPEED_UNITS)

122          return max(-MAX_SPEED_UNITS, min(MAX_SPEED_UNITS, units))

123

124      def set_speeds(self, left: float, right: float) -> None:

125          # Proven convention: right side positive, left side negative = forward.

126          left_units  = -self._pct_to_units(left)

127          right_units =  self._pct_to_units(right)

128

129          self._lf.set_speed(left_units)

130          self._lr.set_speed(left_units)

131          self._rf.set_speed(right_units)

132          self._rr.set_speed(right_units)

133

134          log.debug(f"[BLDC] L={left_units}  R={right_units}")

135

136      def stop(self) -> None:

137          for m in (self._lf, self._lr, self._rf, self._rr):

138              m.set_speed(0)

139

140      def cleanup(self) -> None:

141          self.stop()

142          for m in (self._lf, self._lr, self._rf, self._rr):

143              m.clear_faults()

144          self._i2c.deinit()

145

146

147  # ---------------------------------------------------------------------------

148  # PID controller (unchanged)

149  # ---------------------------------------------------------------------------

150

151  class PIDController:

152      """

153      Line-following PID. error is normalised -1..1 from LineDetector.

154      Output is a turn value passed to MotorDriver.drive().

155      """

156

157      def __init__(self, kp: float = 0.40, ki: float = 0.001, kd: float = 0.15):

158          self.kp = kp

159          self.ki = ki

160          self.kd = kd

161          self._integral   = 0.0

162          self._prev_error = 0.0

163          self._last_time  = time.monotonic()

164          self.integral_max = 100.0

165

166      def compute(self, error: float) -> float:

167          now = time.monotonic()

168          dt  = max(now - self._last_time, 0.001)

169

170          self._integral += error * dt

171          self._integral  = max(-self.integral_max,

172                                min(self.integral_max, self._integral))

173

174          derivative = (error - self._prev_error) / dt

175          output = (self.kp * error

176                  + self.ki * self._integral

177                  + self.kd * derivative)

178

179          self._prev_error = error

180          self._last_time  = now

181          return output

182

183      def reset(self) -> None:

184          self._integral   = 0.0

185          self._prev_error = 0.0

186          self._last_time  = time.monotonic()

187

188

189  # ---------------------------------------------------------------------------

190  # Stub driver for testing without hardware

191  # ---------------------------------------------------------------------------

192

193  class StubMotorDriver(MotorDriver):

194      """Prints commands instead of driving hardware. Safe on any machine."""

195

196      def __init__(self, verbose: bool = True):

197          self.verbose     = verbose

198          self.last_left   = 0.0

199          self.last_right  = 0.0

200

201      def set_speeds(self, left: float, right: float) -> None:

202          self.last_left  = left

203          self.last_right = right

204          if self.verbose:

205              print(f"  [motors] L={left:+6.1f}  R={right:+6.1f}")

206

207      def cleanup(self) -> None:

208          pass

209

## control/state_machine.py

Control loop and states

state_machine.py is where the robot actually decides what to do. The run method loops fifty times a second. On each pass it takes a snapshot of the latest camera results and calls the handler for whatever state the robot is currently in.

The six states are LINE_FOLLOWING, RECOVERING, LOST, INTERSECTION, MARKER_TURN and OBSTACLE_AVOID. A PID controller keeps the robot centred on the line while driving forward. Turns at markers and the arc around an obstacle are handled with a mix of a timer and closed loop checks, so the robot exits early once the line is back in view instead of always waiting for a fixed time to pass.

1  """

2  state_machine.py

3  ================

4  Main control loop at 50 Hz. Implements the line-following flowchart.

5

6  States:

7      LINE_FOLLOWING  - PID on line error

8      RECOVERING      - Line in mid/far ROI only (gap / Logical Pool 2)

9      LOST            - No line anywhere, slow rotate to search

10      INTERSECTION    - Cross-line detected, check green marker

11      MARKER_TURN     - Executing marker-directed turn

12      OBSTACLE_AVOID  - Arcing around obstacle

13

14  Endzone/silver-tape handling and the DONE hand-off are removed for now -

15  the claw isn't ready. The robot just keeps line-following/obstacle-avoiding/

16  marker-turning indefinitely until stopped externally (Ctrl-C, or the

17  switch - see switch_watcher.py).

18  """

19

20  import time

21  import logging

22  from enum import Enum, auto

23  from control.shared_vision import SharedVision

24  from control.motors import MotorDriver, PIDController

25  from vision.line_detector import LineState

26

27  log = logging.getLogger(__name__)

28

29  # ---------------------------------------------------------------------------

30  # Tunables

31  # ---------------------------------------------------------------------------

32  BASE_SPEED           = 80.0

33  RECOVERY_SPEED       = 35.0

34  SEARCH_ROTATE_SPEED  = 28.0

35  OBSTACLE_ARC_SPEED   = 40.0

36

37  OBSTACLE_THRESHOLD_CM   = 18.0

38  OBSTACLE_ARC_DURATION_S = 3.5   # MAX arc time - normally exits early on line reacquisition

39  OBSTACLE_ARC_MIN_S      = 1.2   # min arc time before reacquisition exit is allowed

40                                   # (must be long enough to have left the original line)

41  MOTOR_ERROR_CHECK_S     = 1.0   # how often to poll BLDC ERROR1/ERROR2 registers

42

43  MARKER_TURN_DURATION_S  = 0.35

44  MARKER_TURN_SPEED       = 38.0

45  U_TURN_DURATION_S       = 0.70   # tune on floor - should be ~2x MARKER_TURN_DURATION_S

46  MARKER_COOLDOWN_S       = 1.5    # ignore markers for this long after a turn completes

47

48  LOST_SEARCH_TIMEOUT_S   = 4.0

49

50  LOOP_HZ    = 50

51  LOOP_SLEEP = 1.0 / LOOP_HZ

52

53

54  # ---------------------------------------------------------------------------

55  # Robot states

56  # ---------------------------------------------------------------------------

57

58  class RobotState(Enum):

59      LINE_FOLLOWING  = auto()

60      RECOVERING      = auto()

61      LOST            = auto()

62      INTERSECTION    = auto()

63      MARKER_TURN     = auto()

64      OBSTACLE_AVOID  = auto()

65

66

67  # ---------------------------------------------------------------------------

68  # State machine

69  # ---------------------------------------------------------------------------

70

71  class StateMachine:

72

73      def __init__(self, vision: SharedVision, motors: MotorDriver):

74          self.vision = vision

75          self.motors = motors

76          self.pid = PIDController(kp=0.55, ki=0.001, kd=0.20)

77          self.state  = RobotState.LINE_FOLLOWING

78          self._state_entered_at   = time.monotonic()

79          self._obstacle_arc_start = 0.0

80          self._lost_since         = 0.0

81          self._lost_timeout_logged = False

82          self._search_direction   = 1.0

83          self._marker_turn_dir    = 1.0

84          self._marker_is_uturn    = False

85          self._marker_cooldown_until = 0.0

86          # True once the marker has been observed ABSENT since the last turn.

87          # A new turn requires BOTH: cooldown elapsed AND marker seen to

88          # disappear at least once - a timer alone re-fires on a marker that

89          # simply stays in view longer than the cooldown.

90          self._marker_rearmed        = True

91          self._marker_turn_start  = 0.0

92          self._running            = False

93

94      def run(self) -> None:

95          log.info("[StateMachine] Starting at %d Hz", LOOP_HZ)

96          self._running = True

97          next_error_check = time.monotonic() + MOTOR_ERROR_CHECK_S

98          can_check_errors = hasattr(self.motors, "check_errors")

99          try:

100              while self._running:

101                  tick_start = time.monotonic()

102                  v = self.vision.snapshot()

103                  self._tick(v)

104                  # Periodic BLDC fault poll (overcurrent/overtemp). A stalled

105                  # M2006 commanded at cruise speed will overheat - this is the

106                  # only place the ERROR registers are ever read.

107                  if can_check_errors and tick_start >= next_error_check:

108                      try:

109                          self.motors.check_errors()

110                      except Exception as e:

111                          log.warning("[StateMachine] check_errors failed: %s", e)

112                      next_error_check = tick_start + MOTOR_ERROR_CHECK_S

113                  elapsed = time.monotonic() - tick_start

114                  time.sleep(max(0.0, LOOP_SLEEP - elapsed))

115          except KeyboardInterrupt:

116              log.info("[StateMachine] Interrupted")

117          finally:

118              self.motors.stop()

119              log.info("[StateMachine] Motors stopped")

120

121      def stop(self) -> None:

122          self._running = False

123

124      # ------------------------------------------------------------------

125      # Tick

126      # ------------------------------------------------------------------

127

128      def _tick(self, v: SharedVision) -> None:

129          handlers = {

130              RobotState.LINE_FOLLOWING: self._state_line_following,

131              RobotState.RECOVERING:     self._state_recovering,

132              RobotState.LOST:           self._state_lost,

133              RobotState.INTERSECTION:   self._state_intersection,

134              RobotState.MARKER_TURN:    self._state_marker_turn,

135              RobotState.OBSTACLE_AVOID: self._state_obstacle_avoid,

136          }

137          handler = handlers.get(self.state)

138          if handler:

139              handler(v)

140

141      # ------------------------------------------------------------------

142      # State handlers

143      # ------------------------------------------------------------------

144

145      def _state_line_following(self, v):

146          if v.obstacle_detected and v.obstacle_distance_cm < OBSTACLE_THRESHOLD_CM:

147              log.info("[OBSTACLE] Detected at %.1fcm - avoiding", v.obstacle_distance_cm)

148              self._obstacle_arc_start = time.monotonic()

149              self._transition(RobotState.OBSTACLE_AVOID)

150              return

151          if v.line_state == LineState.LOST:

152              self._lost_since = time.monotonic()

153              self._search_direction = 1.0 if v.last_valid_error > 0 else -1.0

154              self._transition(RobotState.LOST)

155              return

156          if v.line_state == LineState.RECOVERING:

157              self._transition(RobotState.RECOVERING)

158              return

159          if not v.green_marker:

160              self._marker_rearmed = True   # marker gone - allow the next one

161          elif self._marker_rearmed and time.monotonic() > self._marker_cooldown_until:

162              log.info("[MARKER] Green marker detected - checking for turn")

163              self._transition(RobotState.INTERSECTION)

164              return

165          turn = self.pid.compute(v.line_error)

166          self.motors.drive(BASE_SPEED, turn)

167

168      def _state_recovering(self, v):

169          if v.obstacle_detected and v.obstacle_distance_cm < OBSTACLE_THRESHOLD_CM:

170              log.info("[OBSTACLE] Detected at %.1fcm - avoiding", v.obstacle_distance_cm)

171              self._obstacle_arc_start = time.monotonic()

172              self._transition(RobotState.OBSTACLE_AVOID)

173              return

174          if v.line_state == LineState.TRACKING:

175              self.pid.reset()

176              self._transition(RobotState.LINE_FOLLOWING)

177              return

178          if v.line_state == LineState.LOST:

179              self._lost_since = time.monotonic()

180              self._search_direction = 1.0 if v.last_valid_error > 0 else -1.0

181              self._transition(RobotState.LOST)

182              return

183          if not v.green_marker:

184              self._marker_rearmed = True   # marker gone - allow the next one

185          elif self._marker_rearmed and time.monotonic() > self._marker_cooldown_until:

186              log.info("[MARKER] Green marker detected - checking for turn")

187              self._transition(RobotState.INTERSECTION)

188              return

189          turn = self.pid.compute(v.line_error)

190          self.motors.drive(RECOVERY_SPEED, turn)

191

192      def _state_lost(self, v):

193          if v.line_state != LineState.LOST:

194              self.pid.reset()

195              self._lost_timeout_logged = False

196              self._transition(RobotState.LINE_FOLLOWING)

197              return

198          if time.monotonic() - self._lost_since > LOST_SEARCH_TIMEOUT_S:

199              # Do NOT exit the program. Under the rules a Lack of Progress means

200              # the Robot Handler repositions the robot at a Start Location and

201              # restarts it (6.4.3 / 2.8.5) - the program must still be alive and

202              # watching for the line when that happens. Stop the motors and wait;

203              # the line reappearing under the camera re-enters LINE_FOLLOWING

204              # via the check at the top of this handler.

205              if not self._lost_timeout_logged:

206                  log.warning("[LOST] Search timeout - motors stopped, waiting "

207                              "for handler reposition (LoP)")

208                  self._lost_timeout_logged = True

209              self.motors.stop()

210              return

211          if self._search_direction > 0:

212              self.motors.rotate_right(SEARCH_ROTATE_SPEED)

213          else:

214              self.motors.rotate_left(SEARCH_ROTATE_SPEED)

215

216      def _state_intersection(self, v):

217          if v.double_marker:

218              # Both sides - U-turn (180°)

219              self.motors.stop()

220              self._marker_turn_dir   = 1.0   # U-turn direction arbitrary, pick right

221              self._marker_is_uturn   = True

222              self._marker_turn_start = time.monotonic()

223              self._transition(RobotState.MARKER_TURN)

224              log.info("[INTERSECTION] Double marker → U-turn")

225          elif v.green_marker:

226              # Single marker - turn toward marker side

227              self.motors.stop()

228              self._marker_turn_dir   = 1.0 if v.green_marker_x > 0 else -1.0

229              self._marker_is_uturn   = False

230              self._marker_turn_start = time.monotonic()

231              direction = "right" if self._marker_turn_dir > 0 else "left"

232              self._transition(RobotState.MARKER_TURN)

233              log.info(f"[INTERSECTION] Single marker x={v.green_marker_x:+.2f} → {direction}")

234          else:

235              self.pid.reset()

236              self._transition(RobotState.LINE_FOLLOWING)

237              log.info("[INTERSECTION] No marker → straight")

238

239

240      def _state_marker_turn(self, v):

241          elapsed  = time.monotonic() - self._marker_turn_start

242          duration = U_TURN_DURATION_S if self._marker_is_uturn else MARKER_TURN_DURATION_S

243

244          if elapsed < duration:

245              if self._marker_turn_dir > 0:

246                  self.motors.rotate_right(MARKER_TURN_SPEED)

247              else:

248                  self.motors.rotate_left(MARKER_TURN_SPEED)

249          else:

250              self.pid.reset()

251              self._marker_cooldown_until = time.monotonic() + MARKER_COOLDOWN_S

252              self._marker_rearmed = False   # require the marker to disappear

253                                              # before another turn can trigger

254              self._transition(RobotState.LINE_FOLLOWING)

255

256      def _state_obstacle_avoid(self, v):

257          elapsed = time.monotonic() - self._obstacle_arc_start

258          # Closed-loop exit (rule 2.4.7: reacquire the line within 30cm of the

259          # obstacle): once past a minimum arc time - long enough to have left

260          # the original line so we don't instantly "reacquire" the line we

261          # were already on - exit as soon as the detector sees the line again.

262          if elapsed >= OBSTACLE_ARC_MIN_S and v.line_state in (

263                  LineState.TRACKING, LineState.RECOVERING):

264              self.pid.reset()

265              self._transition(RobotState.LINE_FOLLOWING)

266              log.info("[OBSTACLE] Line reacquired after %.1fs arc", elapsed)

267              return

268          if elapsed < OBSTACLE_ARC_DURATION_S:

269              self.motors.set_speeds(OBSTACLE_ARC_SPEED, OBSTACLE_ARC_SPEED * 0.3)

270          else:

271              # Max arc time exhausted without seeing the line - fall back to

272              # line following; the LOST handler will take over if needed.

273              self.pid.reset()

274              self._transition(RobotState.LINE_FOLLOWING)

275              log.warning("[OBSTACLE] Arc timed out without reacquiring line")

276

277      # ------------------------------------------------------------------

278

279      def _transition(self, new_state: RobotState) -> None:

280          if new_state != self.state:

281              log.info("[STATE] %s → %s", self.state.name, new_state.name)

282              self.state = new_state

283              self._state_entered_at = time.monotonic()

284

## control/shared_vision.py

Shared state between threads

shared_vision.py is a small data container shared between three threads that all run at the same time: the downward line camera, the forward obstacle camera and the main control loop. Each camera thread writes its latest result into this object, and the control loop reads a snapshot of it once per tick.

A lock protects every read and write so one thread can never see a half updated value from another. snapshot() returns a plain copy of the current values, which is what the state machine actually works from during a tick.

1  """

2  shared_vision.py

3  ================

4  Thread-safe container that vision threads write to and the control loop reads from.

5  """

6  import threading

7  from dataclasses import dataclass, field

8  from vision.line_detector import LineState

9

10  @dataclass

11  class SharedVision:

12      line_state:           LineState = LineState.LOST

13      line_error:           float     = 0.0

14      line_confidence:      float     = 0.0

15      line_width_px:        float     = 0.0

16      intersection:         bool      = False

17      green_marker:         bool      = False

18      green_marker_x:       float     = 0.0

19      double_marker:        bool      = False

20      last_valid_error:     float     = 0.0

21      obstacle_detected:    bool      = False

22      obstacle_distance_cm: float     = 999.0

23      lock: threading.Lock = field(default_factory=threading.Lock, repr=False)

24

25      def update_line(self, result) -> None:

26          with self.lock:

27              self.line_state       = result.state

28              self.line_error       = result.error

29              self.line_confidence  = result.confidence

30              self.line_width_px    = result.line_width_px

31              self.intersection     = result.intersection

32              self.green_marker     = result.green_marker

33              self.green_marker_x   = result.green_marker_x

34              self.double_marker    = result.double_marker

35              self.last_valid_error = result.last_error

36

37      def update_forward(self, obstacle: bool, distance_cm: float) -> None:

38          with self.lock:

39              self.obstacle_detected    = obstacle

40              self.obstacle_distance_cm = distance_cm

41

42      def snapshot(self) -> "SharedVision":

43          with self.lock:

44              return SharedVision(

45                  line_state           = self.line_state,

46                  line_error            = self.line_error,

47                  line_confidence       = self.line_confidence,

48                  line_width_px         = self.line_width_px,

49                  intersection          = self.intersection,

50                  green_marker          = self.green_marker,

51                  green_marker_x        = self.green_marker_x,

52                  double_marker         = self.double_marker,

53                  last_valid_error      = self.last_valid_error,

54                  obstacle_detected     = self.obstacle_detected,

55                  obstacle_distance_cm  = self.obstacle_distance_cm,

56              )

57

## vision/line_detector.py

Line and marker detection

line_detector.py processes each frame from the downward camera. detect() converts the frame to a binary black and white image using adaptive thresholding, which adjusts to local brightness rather than a single fixed cutoff, so it keeps working as lighting changes across the course.

It looks for the line in three horizontal bands, near, mid and far, so it can tell the difference between the line briefly being out of the closest band and the line being genuinely lost. It also looks for green markers, working out which side of the line a single marker is on, and detecting when two markers appear at once, which signals a U-turn rather than a normal turn.

1  """

2  line_detector.py

3  ================

4  Robust black-line detector for RoboCup Junior Rescue Line (Open division).

5  Designed for Raspberry Pi 5 + downward-facing Pi Camera Module V2.

6  """

7

8  import cv2

9  import numpy as np

10  from dataclasses import dataclass, field

11  from enum import Enum, auto

12  from typing import Optional

13

14  # ---------------------------------------------------------------------------

15  # Tunables

16  # ---------------------------------------------------------------------------

17  ROI_NEAR  = (0.75, 1.00)

18  ROI_MID   = (0.50, 0.75)

19  ROI_FAR   = (0.25, 0.50)

20

21  ADAPTIVE_BLOCK  = 51

22  ADAPTIVE_C      = 8

23

24  MIN_LINE_AREA      = 300

25  LINE_WIDTH_MIN_PX  = 6

26  LINE_WIDTH_MAX_PX  = 200

27

28  LOST_PATIENCE_FRAMES = 8

29

30  INTERSECTION_SPAN_FRACTION = 0.45

31

32  GREEN_H_LOW,  GREEN_H_HIGH  = 38,  85

33  GREEN_S_LOW,  GREEN_S_HIGH  = 60, 255

34  GREEN_V_LOW,  GREEN_V_HIGH  = 40, 255

35  GREEN_MIN_AREA_PX           = 400

36

37

38  # ---------------------------------------------------------------------------

39  # Calibration persistence - the constants above are what calibrate.py tunes

40  # and saves to calibration.json. load_calibration() applies a saved file back

41  # onto this module so a real run picks up on-site tuning, not just defaults.

42  # ---------------------------------------------------------------------------

43

44  def load_calibration(path: str = "calibration.json") -> dict:

45      """

46      Load a calibration.json saved by calibrate.py and apply it to this

47      module's constants. Call once at startup before creating LineDetector.

48      Returns the loaded dict, or {} if no file was found.

49      """

50      import json, os

51      if not os.path.exists(path):

52          print(f"[LineDetector] No {path} found - using built-in defaults")

53          return {}

54

55      with open(path, "r") as f:

56          values = json.load(f)

57

58      applied = {}

59      for key, value in values.items():

60          if key in globals():

61              globals()[key] = value

62              applied[key] = value

63          # darkness_bias isn't a module constant - it's a per-instance value

64          # set by LineDetector.calibrate() itself, so it's intentionally skipped.

65

66      print(f"[LineDetector] Loaded calibration from {path}: {applied}")

67      return applied

68

69

70  # ---------------------------------------------------------------------------

71  # Data types

72  # ---------------------------------------------------------------------------

73

74  class LineState(Enum):

75      TRACKING       = auto()

76      RECOVERING     = auto()

77      LOST           = auto()

78      INTERSECTION   = auto()

79

80

81  @dataclass

82  class LineResult:

83      state:          LineState

84      error:          float = 0.0

85      error_px:       float = 0.0

86      confidence:     float = 0.0

87      line_width_px:  float = 0.0

88      intersection:   bool  = False

89      green_marker:   bool  = False

90      green_marker_x: float = 0.0

91      double_marker:  bool  = False   # both sides → U-turn

92      last_error:     float = 0.0

93      debug_frame:    Optional[np.ndarray] = field(default=None, repr=False)

94

95

96  # ---------------------------------------------------------------------------

97  # Main detector

98  # ---------------------------------------------------------------------------

99

100  class LineDetector:

101

102      def __init__(self, frame_width: int = 640, frame_height: int = 480, debug: bool = False):

103          self.fw = frame_width

104          self.fh = frame_height

105          self.debug = debug

106          self._darkness_bias   = 0

107          self._tile_brightness = 200.0

108          self._lost_counter    = 0

109          self._last_error      = 0.0

110          self._last_state      = LineState.LOST

111

112      def calibrate(self, frame: np.ndarray) -> dict:

113          gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

114          h, w = gray.shape

115          sz = 30

116          corners = [

117              gray[:sz, :sz], gray[:sz, w-sz:],

118              gray[h-sz:, :sz], gray[h-sz:, w-sz:],

119          ]

120          bg_brightness = float(np.mean([c.mean() for c in corners]))

121          self._tile_brightness = bg_brightness

122          strip = gray[int(h*0.7):int(h*0.9), int(w*0.3):int(w*0.7)]

123          line_brightness = float(strip.min())

124          contrast = bg_brightness - line_brightness

125          if contrast < 40:

126              self._darkness_bias = 4

127          elif contrast < 70:

128              self._darkness_bias = 0

129          else:

130              self._darkness_bias = -3

131          result = {

132              "bg_brightness": round(bg_brightness, 1),

133              "line_brightness": round(line_brightness, 1),

134              "contrast": round(contrast, 1),

135              "darkness_bias": self._darkness_bias,

136          }

137          print(f"[LineDetector] Calibration: {result}")

138          return result

139

140      def detect(self, frame: np.ndarray) -> LineResult:

141          if frame is None or frame.size == 0:

142              return LineResult(state=LineState.LOST, last_error=self._last_error)

143          h, w = frame.shape[:2]

144          debug_frame  = frame.copy() if self.debug else None

145          binary       = self._threshold(frame)

146          near = self._analyse_roi(binary, ROI_NEAR, w, h, debug_frame, (0, 255, 0))

147          mid  = self._analyse_roi(binary, ROI_MID,  w, h, debug_frame, (0, 200, 255))

148          far  = self._analyse_roi(binary, ROI_FAR,  w, h, debug_frame, (255, 150, 0))

149          # Marker side is judged relative to the LINE, not the frame centre -

150          # if the robot approaches an intersection offset from the line, a

151          # frame-centre reference can put the marker on the wrong side.

152          line_cx = near["cx"] if near is not None else (mid["cx"] if mid is not None else None)

153          green_marker, green_marker_x, double_marker = self._detect_green_marker(frame, debug_frame, line_cx)

154          intersection = self._detect_intersection(binary, ROI_MID, w, h, debug_frame)

155          return self._build_result(

156              near, mid, far, intersection,

157              green_marker, green_marker_x, double_marker,

158              w, debug_frame

159          )

160

161      def _threshold(self, frame):

162          lab    = cv2.cvtColor(frame, cv2.COLOR_BGR2LAB)

163          L      = lab[:, :, 0]

164          c      = ADAPTIVE_C + self._darkness_bias

165          binary = cv2.adaptiveThreshold(

166              L, 255, cv2.ADAPTIVE_THRESH_GAUSSIAN_C,

167              cv2.THRESH_BINARY_INV, ADAPTIVE_BLOCK, c

168          )

169          k_close = cv2.getStructuringElement(cv2.MORPH_RECT, (5, 5))

170          binary  = cv2.morphologyEx(binary, cv2.MORPH_CLOSE, k_close, iterations=1)

171          k_open  = cv2.getStructuringElement(cv2.MORPH_RECT, (3, 3))

172          binary  = cv2.morphologyEx(binary, cv2.MORPH_OPEN,  k_open,  iterations=1)

173          binary  = self._mask_green(frame, binary)

174          # Zero out top 25% too close, only sees robot chassis

175          binary[:int(binary.shape[0] * 0.25), :] = 0

176          return binary

177

178      def _mask_green(self, frame, binary):

179          hsv  = cv2.cvtColor(frame, cv2.COLOR_BGR2HSV)

180          mask = cv2.inRange(

181              hsv,

182              (GREEN_H_LOW,  GREEN_S_LOW,  GREEN_V_LOW),

183              (GREEN_H_HIGH, GREEN_S_HIGH, GREEN_V_HIGH)

184          )

185          binary[mask > 0] = 0

186          return binary

187

188      def _analyse_roi(self, binary, roi_frac, frame_w, frame_h, debug_frame, colour):

189          y_start = int(frame_h * roi_frac[0])

190          y_end   = int(frame_h * roi_frac[1])

191          roi     = binary[y_start:y_end, :]

192          contours, _ = cv2.findContours(roi, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE)

193          best      = None

194          best_area = 0

195          for cnt in contours:

196              area = cv2.contourArea(cnt)

197              if area < MIN_LINE_AREA:

198                  continue

199              x, y, w, h = cv2.boundingRect(cnt)

200              if not (LINE_WIDTH_MIN_PX <= w <= LINE_WIDTH_MAX_PX):

201                  continue

202              if area > best_area:

203                  best_area = area

204                  M  = cv2.moments(cnt)

205                  cx = int(M["m10"] / M["m00"]) if M["m00"] > 0 else x + w // 2

206                  best = {

207                      "cx": cx, "cy": y + h // 2 + y_start,

208                      "area": area, "width_px": w,

209                      "confidence": min(1.0, area / 6000),

210                      "bbox": (x, y + y_start, w, h),

211                  }

212          if best and self.debug and debug_frame is not None:

213              x, y, w, h = best["bbox"]

214              cv2.rectangle(debug_frame, (x, y), (x+w, y+h), colour, 2)

215              cv2.circle(debug_frame, (best["cx"], best["cy"]), 5, colour, -1)

216          return best

217

218      def _detect_intersection(self, binary, roi_frac, frame_w, frame_h, debug_frame):

219          return False

220

221      def _detect_green_marker(self, frame, debug_frame, line_cx=None):

222          hsv  = cv2.cvtColor(frame, cv2.COLOR_BGR2HSV)

223          mask = cv2.inRange(

224              hsv,

225              (GREEN_H_LOW,  GREEN_S_LOW,  GREEN_V_LOW),

226              (GREEN_H_HIGH, GREEN_S_HIGH, GREEN_V_HIGH)

227          )

228          h, w = mask.shape

229          # Only trigger on markers in the NEAR (bottom) half of the frame -

230          # i.e. when the marker is under/just ahead of the robot. Rule 2.3.1.4

231          # places markers immediately BEFORE the intersection, so triggering on

232          # far-field sightings made the robot pivot mid-tile, well before the

233          # intersection (→ Lack of Progress under 6.4.1.7). With the near-field

234          # gate, by the time the marker fills the bottom half the robot is at

235          # the intersection and the turn happens in the right place.

236          mask[:h // 2, :] = 0

237

238          # Reference x for "left/right of the line": the detected line centre

239          # if we have one, else the frame centre as a fallback.

240          ref_x = float(line_cx) if line_cx is not None else w / 2

241

242          contours, _ = cv2.findContours(mask, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE)

243

244          valid_markers = []   # (area, x_norm relative to line)

245          for cnt in contours:

246              area = cv2.contourArea(cnt)

247              if area < GREEN_MIN_AREA_PX:

248                  continue

249              x, y, cw, ch = cv2.boundingRect(cnt)

250              if 0.4 < (cw / max(ch, 1)) < 2.5:

251                  marker_cx     = x + cw / 2

252                  marker_x_norm = (marker_cx - ref_x) / (w / 2)

253                  valid_markers.append((area, marker_x_norm))

254                  if self.debug and debug_frame is not None:

255                      cv2.rectangle(debug_frame, (x, y), (x+cw, y+ch), (0, 200, 0), 2)

256                      cv2.putText(debug_frame, f"M x={marker_x_norm:+.2f}",

257                                  (x, y-5), cv2.FONT_HERSHEY_SIMPLEX, 0.45, (0, 200, 0), 2)

258

259          if len(valid_markers) == 0:

260              return False, 0.0, False

261

262          # Check for double marker - one on each side of the line

263          has_left  = any(x < -0.15 for _, x in valid_markers)

264          has_right = any(x >  0.15 for _, x in valid_markers)

265          double_marker = has_left and has_right

266

267          if double_marker:

268              if self.debug and debug_frame is not None:

269                  cv2.putText(debug_frame, "DOUBLE MARKER → U-TURN",

270                              (10, 40), cv2.FONT_HERSHEY_SIMPLEX, 0.6, (0, 0, 255), 2)

271              return True, 0.0, True   # x=0.0 irrelevant for u-turn

272

273          # Single marker - largest area wins (contour enumeration order is

274          # arbitrary; a small green false-positive must not outvote the real

275          # 40x40mm marker).

276          valid_markers.sort(key=lambda t: -t[0])

277          best_x = valid_markers[0][1]

278          return True, best_x, False

279

280      def _build_result(self, near, mid, far, intersection,

281                        green_marker, green_marker_x, double_marker,

282                        frame_w, debug_frame):

283          half_w = frame_w / 2

284

285          def norm_error(r):

286              raw = r["cx"] - half_w

287              return float(np.clip(raw / half_w, -1.0, 1.0)), float(raw)

288

289          if near is not None:

290              error_norm, error_px = norm_error(near)

291              self._lost_counter = 0

292              self._last_error   = error_norm

293              state = LineState.INTERSECTION if intersection else LineState.TRACKING

294              self._last_state   = state

295              if self.debug and debug_frame is not None:

296                  col = (0, 255, 0) if state == LineState.TRACKING else (0, 100, 255)

297                  cv2.putText(debug_frame, f"err={error_norm:+.3f}  {state.name}",

298                              (10, 20), cv2.FONT_HERSHEY_SIMPLEX, 0.55, col, 2)

299              return LineResult(

300                  state=state,

301                  error=error_norm,

302                  error_px=error_px,

303                  confidence=near["confidence"],

304                  line_width_px=near["width_px"],

305                  intersection=intersection,

306                  green_marker=green_marker,

307                  green_marker_x=green_marker_x,

308                  double_marker=double_marker,

309                  last_error=self._last_error,

310                  debug_frame=debug_frame

311              )

312

313          recovery = mid if mid is not None else far

314          if recovery is not None:

315              error_norm, error_px = norm_error(recovery)

316              self._lost_counter = 0

317              self._last_error   = error_norm

318              self._last_state   = LineState.RECOVERING

319              if self.debug and debug_frame is not None:

320                  cv2.putText(debug_frame, f"RECOVERING err={error_norm:+.3f}",

321                              (10, 20), cv2.FONT_HERSHEY_SIMPLEX, 0.55, (0, 200, 255), 2)

322              return LineResult(

323                  state=LineState.RECOVERING,

324                  error=error_norm,

325                  error_px=error_px,

326                  confidence=recovery["confidence"] * 0.6,

327                  line_width_px=recovery["width_px"],

328                  intersection=intersection,

329                  green_marker=green_marker,

330                  green_marker_x=green_marker_x,

331                  double_marker=double_marker,

332                  last_error=self._last_error,

333                  debug_frame=debug_frame

334              )

335

336          self._lost_counter += 1

337          if self._lost_counter >= LOST_PATIENCE_FRAMES:

338              state = LineState.LOST

339          else:

340              state = self._last_state if self._last_state != LineState.LOST else LineState.RECOVERING

341          if self.debug and debug_frame is not None:

342              cv2.putText(debug_frame, f"LOST ({self._lost_counter}/{LOST_PATIENCE_FRAMES})",

343                          (10, 20), cv2.FONT_HERSHEY_SIMPLEX, 0.55, (0, 0, 255), 2)

344          self._last_state = state

345          return LineResult(

346              state=state,

347              error=self._last_error,

348              error_px=self._last_error * half_w,

349              confidence=0.0,

350              intersection=False,

351              green_marker=green_marker,

352              green_marker_x=green_marker_x,

353              double_marker=double_marker,

354              last_error=self._last_error,

355              debug_frame=debug_frame

356          )

## vision/vision_thread.py

Camera threads

vision_thread.py runs the two Pi cameras in their own background threads using picamera2, so the control loop is never stuck waiting on a camera frame to arrive. The downward camera runs at thirty frames a second for line following, and the forward camera runs at fifteen frames a second, which is enough for spotting obstacles.

The forward camera estimates distance to an obstacle from how large its outline appears in the frame, using a reference value, AREA_AT_18CM, measured by placing a real obstacle eighteen centimetres from the camera and recording the contour area at that distance with calibrate_obstacle.py. A larger contour than the reference means the obstacle is closer than eighteen centimetres, and a smaller one means it is further away.

1  """

2  vision_thread.py

3  ================

4  Camera threads for two Pi Camera Module V2 units connected via CSI.

5

6  Camera assignment (based on physical robot):

7      CAM1 - downward facing  → line following   (LINE_CAM = 1)

8      CAM0 - forward facing   → obstacle detection (FWD_CAM = 0)

9

10  Uses picamera2 (not cv2.VideoCapture) because Pi Camera Modules are

11  CSI cameras, not USB cameras - they don't appear as /dev/videoX devices

12  and won't open with VideoCapture.

13

14  Install picamera2 if not present:

15      sudo apt install python3-picamera2 -y

16

17  If cameras are on opposite ports to what's listed, swap LINE_CAM and FWD_CAM.

18  """

19

20  import threading

21  import logging

22  import time

23  import numpy as np

24  from vision.line_detector import LineDetector

25  from control.shared_vision import SharedVision

26

27  log = logging.getLogger(__name__)

28

29  # Which CSI port each camera is on

30  LINE_CAM = 0   # downward facing - line following

31  FWD_CAM  = 1   # forward facing  - obstacle detection

32

33  FRAME_WIDTH  = 640

34  FRAME_HEIGHT = 480

35  LINE_CAM_FPS = 30

36  FWD_CAM_FPS  = 15   # obstacles don't need high frame rate

37

38  # --debug frame dump (headless inspection): every Nth debug frame is written

39  # to DEBUG_DIR, cycling through DEBUG_KEEP files so disk use is bounded.

40  DEBUG_DIR        = "/tmp/rescue_debug"

41  DEBUG_EVERY_N    = 15   # at 30fps ≈ 2 saves/sec

42  DEBUG_KEEP       = 20   # ring buffer of files

43

44  # Obstacle detection: contour area at a known distance (calibrate on your

45  # robot - see calibrate_obstacle.py). Anchored at 18cm to match

46  # OBSTACLE_THRESHOLD_CM in state_machine.py, since that's the distance the

47  # avoidance decision actually triggers at - the area↔distance formula below

48  # is an approximation, most accurate near wherever it's anchored, so it's

49  # most accurate right where the decision matters instead of some unrelated

50  # distance.

51  AREA_AT_18CM = 3696.0   # measured via calibrate_obstacle.py: 49/49 frames, median 3696, std dev 408 (~11%)

52

53

54  class VisionThread:

55

56      def __init__(

57          self,

58          shared:            SharedVision,

59          line_cam_index:    int  = LINE_CAM,

60          fwd_cam_index:     int  = FWD_CAM,

61          debug:             bool = False,

62      ):

63          self.shared         = shared

64          self.debug          = debug

65          self._line_cam_idx  = line_cam_index

66          self._fwd_cam_idx   = fwd_cam_index

67          self._running       = False

68          self._cal_frame     = None

69          self._dbg_counter   = 0

70

71          self.detector = LineDetector(FRAME_WIDTH, FRAME_HEIGHT, debug=debug)

72

73          self._line_thread = threading.Thread(

74              target=self._line_loop, name="line-cam", daemon=True

75          )

76          self._fwd_thread = threading.Thread(

77              target=self._fwd_loop, name="fwd-cam", daemon=True

78          )

79

80      def start(self, calibration_frame=None) -> None:

81          self._running   = True

82          self._cal_frame = calibration_frame

83          self._line_thread.start()

84          self._fwd_thread.start()

85          log.info("[Vision] Started - line=CAM%d  fwd=CAM%d",

86                   self._line_cam_idx, self._fwd_cam_idx)

87

88      def stop(self) -> None:

89          self._running = False

90          self._line_thread.join(timeout=3.0)

91          self._fwd_thread.join(timeout=3.0)

92          log.info("[Vision] Stopped")

93

94      # ------------------------------------------------------------------

95      # Line camera loop - Pi Camera Module V2 via picamera2

96      # ------------------------------------------------------------------

97

98      def _line_loop(self) -> None:

99          try:

100              from picamera2 import Picamera2

101          except ImportError:

102              log.error("[line-cam] picamera2 not installed. Run: sudo apt install python3-picamera2 -y")

103              return

104

105          cam = Picamera2(self._line_cam_idx)

106          config = cam.create_preview_configuration(

107              main={"size": (FRAME_WIDTH, FRAME_HEIGHT), "format": "BGR888"},

108              controls={"FrameRate": LINE_CAM_FPS}

109          )

110          cam.configure(config)

111          cam.start()

112          log.info("[line-cam] Camera started on CAM%d", self._line_cam_idx)

113

114          calibrated = False

115

116          try:

117              while self._running:

118                  # capture_array returns BGR numpy array directly

119                  frame = cam.capture_array()

120

121                  if not calibrated:

122                      ref = self._cal_frame if self._cal_frame is not None else frame

123                      self.detector.calibrate(ref)

124                      calibrated = True

125                      log.info("[line-cam] Calibrated")

126

127                  result = self.detector.detect(frame)

128                  self.shared.update_line(result)

129

130                  if self.debug and result.debug_frame is not None:

131                      # Save debug frames to disk periodically for headless

132                      # inspection (imshow won't work over SSH). Ring buffer of

133                      # DEBUG_KEEP files in DEBUG_DIR, ~2 frames/sec.

134                      self._dbg_counter += 1

135                      if self._dbg_counter % DEBUG_EVERY_N == 0:

136                          idx = (self._dbg_counter // DEBUG_EVERY_N) % DEBUG_KEEP

137                          import cv2, os

138                          os.makedirs(DEBUG_DIR, exist_ok=True)

139                          cv2.imwrite(os.path.join(DEBUG_DIR, f"line_{idx:02d}.jpg"),

140                                      result.debug_frame)

141

142          finally:

143              cam.stop()

144              log.info("[line-cam] Camera stopped")

145

146      # ------------------------------------------------------------------

147      # Forward camera loop - Pi Camera Module V2 via picamera2

148      # ------------------------------------------------------------------

149

150      def _fwd_loop(self) -> None:

151          try:

152              from picamera2 import Picamera2

153          except ImportError:

154              log.error("[fwd-cam] picamera2 not installed. Run: sudo apt install python3-picamera2 -y")

155              return

156

157          cam = Picamera2(self._fwd_cam_idx)

158          config = cam.create_preview_configuration(

159              main={"size": (FRAME_WIDTH, FRAME_HEIGHT), "format": "BGR888"},

160              controls={"FrameRate": FWD_CAM_FPS}

161          )

162          cam.configure(config)

163          cam.start()

164          log.info("[fwd-cam] Camera started on CAM%d", self._fwd_cam_idx)

165

166          try:

167              import cv2

168              while self._running:

169                  frame = cam.capture_array()

170                  obstacle, distance_cm = self._detect_obstacle(frame, cv2)

171                  self.shared.update_forward(obstacle, distance_cm)

172

173          finally:

174              cam.stop()

175              log.info("[fwd-cam] Camera stopped")

176

177      def _detect_obstacle(self, frame: np.ndarray, cv2) -> tuple:

178          """

179          Detect obstacles in the forward camera frame.

180

181          Obstacles (§2.4.5): min 150mm high, base max 150mm diameter,

182          placed min 250mm from field edge. They are large, blocky, and dark

183          relative to the white tile background.

184

185          Returns (obstacle_detected: bool, distance_cm: float)

186          """

187          gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

188          h, w = gray.shape

189

190          # Only look in lower half - obstacles sit on the floor

191          roi = gray[h // 2:, :]

192

193          # Adaptive threshold - more robust than fixed value under venue lighting

194          binary = cv2.adaptiveThreshold(

195              roi, 255,

196              cv2.ADAPTIVE_THRESH_GAUSSIAN_C,

197              cv2.THRESH_BINARY_INV,

198              51, 8

199          )

200

201          contours, _ = cv2.findContours(

202              binary, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE

203          )

204

205          obstacle    = False

206          distance_cm = 999.0

207

208          for cnt in contours:

209              area = cv2.contourArea(cnt)

210              if area < 1500:

211                  continue

212

213              x, y, bw, bh = cv2.boundingRect(cnt)

214

215              # Must be tall enough - obstacles are min 150mm, debris is max 3mm

216              # At typical camera height, 150mm obstacle ≈ 30+ px tall

217              if bh < 30:

218                  continue

219

220              # Must be blocky - not the line itself

221              if bw < 20:

222                  continue

223

224              # Rough distance: larger contour area = closer

225              est_cm = (AREA_AT_18CM / area) * 18.0

226              if est_cm < distance_cm:

227                  distance_cm = est_cm

228                  obstacle    = True

229

230          return obstacle, distance_cm

231